GOVERNOR-IN-COUNCIL

MANIPUR STATE POWER DISTRIBUTION COMPANY LIMITED (MSPDCL)

NOTIFICATION

Place…… Date……….

Memo No: - The Government of India issued the Energy Conservation Building Code, 2017, under clause (p) of section 14 of Energy Conservation Act, 2001. Clause (a) of section 15 of the Energy Conservation Act, 2001 empowers the State Government to amend the Energy Conservation Building Code, in consultation with the Bureau of Energy Efficiency, to suit the regional and local climatic conditions and notify Energy Conservation Building Code with respect to use of energy in the buildings.

Now therefore, in exercise of the powers conferred by clause (a) of Section 15 and section 57, read with clause (1) of sub-section (2) of section 56, clause (d) of sub-section (2) of section 13 and clause (p) of section 14 of the Energy Conservation Act, 2001 (Central Act 52 of 2001), the Governor-in-Council of Manipur hereby makes the following rules to notify the Manipur Energy Conservation Building Code (MECBC) applicable to the State of Manipur, namely;

- Short Title and Commencement –

- These rules may be called the Manipur Energy Conservation Building Code (MECBC) Rules, 2025 and subsequent updations/clarifications/amendments made from time to time as adopted by the Bureau of Energy efficiency (BEE).

- They shall come into force on the date of their publication in the State Gazette.

- Definition –

- In these rules, unless the context otherwise requires, –

- “Act” means the Energy Conservation Act, 2001 (Central Act 52 of 2001); and its amendments from time to time;

- “Authority having jurisdiction” means the concerned department i.e., local body or organization or authority, empowered by relevant law to sanction building plans; to inspect the building; and/or to issue the completion/occupation certificate to the owners or their authorized agents; or an organisation, office, or individual responsible for enforcing the requirements of the code or standard, or for approving equipment, materials, an installation, or a procedure;

- “best practices” means those measures that –

allow for optimisation of efficiencies in the identified components and systems to enhance the energy efficiency of the building; or

- reduce the cost of construction having regard to the safety, stability of the building structure, health and environmental provisions of Central laws or Manipur State Government laws; and

- includes energy conservation measures approved by the Manipur Energy Conservation Building Code Implementation Committee or Manipur Energy Conservation Building Code Compliant Technical Grievances Redressal Committee;

- “building complex” means a building or group of buildings constructed in a contiguous area for business, commercial, institutional purposes or assembly of buildings under the single ownership of individuals or group of individuals or under the name of a co-operative group society or on lease and sold as shops or office space or space for other commercial purposes;

- “built-up area” means the total covered areas on all floors of a building from the basement to all storeys covered by walls and parapet measured at the floor levels excluding parking;

- “Bureau” means Bureau of Energy Efficiency (BEE);

- “bye-laws” means the building bye-laws framed by Manipur Government or any authority under its control to regulate the building activities in its areas falling in the jurisdiction of-

- all Municipal authorities or Committees or Councils;

- all Metropolitan area or Nagar Panchayats;

- all areas covered under the Development or Planning Authorities;

under various development plans notified by Manipur Government and enforced by such authority in its jurisdiction in which the Manipur Energy Conservation Building Code compliant building shall be located and includes any regulation or rule framed by the Manipur Government or any other authority having jurisdiction established by the Manipur Government.

- “Certified Energy Auditor (Building)” means a person who fulfills the eligibility criteria specified in the Energy Conservation (Minimum qualification for Energy Auditors and Energy Managers) Rules, 2006 and has qualified National Examination for Energy Conservation Building Codes Compliance. BEE Empanelled ECBC Expert/ Design Professional or MSPDCL Empanelled ECBC Design Professionals shall be eligible in place of Certified Energy Auditor (Building) till the cadre of Certified Energy Auditor (Building) shall be created by BEE;

- “Code” means the Manipur Energy Conservation Building Code (MECBC) 2025 amended the State Govt./BEE from time to time under the EC Act;

- “compliance documents” means the forms specified in Appendix D of the Code and includes certificates from Empanelled Energy Auditors (Building) to conform compliance with these rules.

- “connected load” means the total of the rated wattage of all equipment, appliances and devices to be installed or installed in the building or part of the building or building complexes in terms of kiloWatt (kW) that will be allocated to all applicants for electric power consumption in respect of the proposed building or building complex, as the case may be, on their completion;

- “construction documents” mean drawings or documents containing information pertaining to building construction processes and approvals, building materials and equipment specification, architectural details required by the authority having jurisdiction;

- “contract demand” means the maximum demand in kiloWatt (kW) or kilo-Volt Ampere (kVA) (within a consumer’s sanctioned load) agreed to be supplied by the electricity provider or utility in the agreement executed between the user and the utility or electricity provider;

- “Empanelled Energy Auditor (Building)” means a firm consisting of the Certified Energy Auditor under Bureau of Energy Efficiency (Certification Procedures for Energy Auditors and Energy Managers) Regulations, 2010 and Certified Energy Auditor (Building), and empanelled with the Bureau;

- “energy conservation measures” mean the measures incorporated in the building design for saving energy, or enhancing comfort in peak electrical or thermal demand, or reducing cooling or heating load covering any element of a component with any other element of the same or other component of the Code and includes any such measure incorporated in the said building design of the proposed or existing building;

- “energy performance index” (EPI) means annual energy consumption of a building in kiloWatt-hours per square meter of the area of the building which shall be calculated as per the following formula:

Energy Performance Index EPI in kWh, = Annual Energy Consumption in kWhTotal built up area excluding storage area and the parking in the basement in m2

- “energy performance index ratio” means the ratio of the energy performance index of the proposed building to the energy performance index of the standard baseline building;

- “establishment” means a business or other organization, or the place where an organization operates and includes a government establishment and private establishment;

- “form” means the forms appended to these rules;

- “owner” means a person, group of persons, a company, a trust, an institute, registered body, State Government or Central Government and its attached or sub-ordinate departments, undertakings and such other agencies or organizations in whose name the property stands registered in the revenue records for the construction of a building or building complex;

- “proposed design” means the computerized design of a building consistent with the actual design of a building which complies with all the requirements of the Code either through prescriptive or whole building performance method;

- “registered architect” means the architects registered with valid membership of the Council of Architecture (CoA), India as prescribed under the Architect’s Act 1972;

- “standard baseline design” means the standard design that complies with all the mandatory and prescriptive requirements of the Code and has the same built-up area of the proposed building;

- “State Designated Agency” means the agency designated under clause (d) of section 15 of the Act, and implies in the context of Manipur State Power Distribution Company Limited (MSPDCL).

- Words and expressions used herein are not defined, but defined in the Act, or in the Code, shall have the meanings respectively assigned to them in the Act or in the said Code.

- Application –

- These rules shall apply to every building, which is used or intended to be used for commercial purposes, having connected load of 50 kiloWatt (kW) or above; or contract demand of 60 kilo-Volt Ampere (kVA) or above; and such building shall cover the following components, namely: -

- Building Envelope;

- Comfort systems and controls (heating, ventilation and air conditioning service hot water system);

- Lighting and controls;

- Electrical and Renewable energy systems;

- Any other system, as may be specified from time to time by the Bureau/ State:

- Building Classification - Any one or more buildings or part of a building with commercial use is classified as per the functional requirements of its design, construction and use. The key classification is as below:

- Hospitality

- Health Care

- Assembly

- Business

- Educational

- Shopping Complex

- Mixed-use Building

- And any other categorization, as may be specified from time to time by Bureau/State

- Provided that these rules shall not apply to equipment, appliances, devices and parts of building that use energy primarily for manufacturing processes:

- Provided further that wherever these rules are in conflict with safety, security, health or environmental codes, or Bureau of Energy Efficiency’s Standard and Labelling for equipment or appliances and Star Rating Program for buildings and if they are more stringent than the requirement of these rules then they shall prevail over these rules:

- Provided also that if any existing building after additions or alterations changes its connected load to 50 kiloWatt (kW) or above or a contract demand of 60 kilo-Volt Ampere (kVA) or above shall comply with the provisions referred to in clauses (i) to (v) of sub-rule I of rule 3 (detailed in section 4 to 7 of the Manipur ECBC book).

- Compliance Mechanism –

- The compliance of energy performance of a building shall be ensured by the owner by following either of the following methods, namely: -

- Prescriptive Method- The building shall comply with the mandatory requirements and prescriptive requirements as specified in the Code for envelope components, comfort systems and controls, lighting and controls, electrical and renewable energy systems;

- Whole Building Performance Method - The building shall comply with all mandatory measures and the requirements specified in the whole building performance method of the Code and the energy performance index of the proposed design under this method shall be the same or less than the energy performance index of the standard baseline design of building as follows:

energy performance index ratio= energy performnace index of proposed designenergy performnace index of standard baseline design≤1

- The summary covering building envelope, comfort systems and controls, lighting and controls, and electrical and renewable energy systems and their checklists under Prescriptive Method and Whole Building Performance Method shall be as specified in the Appendix D of the Code.

- Procedure for erection of Code compliant building –

- Every owner who intends to erect or re-erect a building or make alterations or additions in any building under these rules shall submit to the concerned authority having jurisdiction, an application in Form I accompanied by-

- undertaking of the owner in Form II;

- construction documents duly signed by the registered architect together with an undertaking in Form III;

- construction documents shall ensure –

- compliance with the applicable building bye-laws in force;

- building design incorporates energy conservation measures and best national and international practices having regard to the climatic conditions of the site and specific needs of the building so as to optimise the energy performance index ratio of the building;

- that all the data, building features, identified energy conservation measures under various building components and systems are shown in detail and in the manner specified in the applicable bye-laws;

- the drawing of plan, color of plan, dimensions of plan, scale of plan as per requirements of the applicable bye-laws in force;

- compliance documents covering the construction of components and system of the Code, duly certified by Empanelled Energy Auditors (Building) including the following, namely: -

- energy performance index ratio report in respect of the proposal building at the design stage;

- certificate in Form IV by Empanelled Energy Auditor (Building) certifying the compliance documents as specified in Appendix D of the Code;

- have been scrutinized or verified in respect of the identified energy conservation measures; and

- an application with heading super scribed “Application for permission to erect/re-erect a Manipur Energy Conservation Building Code Compliant Building”, duly signed by the owner seeking building permit from the concerned authority having jurisdiction before starting construction work in respect of the proposed building.

- The authority having jurisdiction may require submission of documents in electronic form or hard copy of the documents, referred to in sub-rule (I) of rule 5.

- The Empanelled Energy Auditors (Building), at the design stage, shall follow the following procedure of inspection, namely: -

- scrutinize the construction documents with respect to -

- floor area;

- window area;

- wall area;

- roof area of the building;

- built-up area of the proposed design of the building;

- scrutinize the Code compliance documents and the check list as specified in the Appendix D of the Code and identify –

- the energy conservation measures that are applicable to the proposed design of building;

- insulation quantities in walls and roof, and the construction assemblies, solar heat gain co-efficient, visible light transmittance and thermal transmittance (U-factor) for window assemblies;

- heating, ventilation and air-conditioning component tables for air-handling equipment, refrigeration equipment, condensing equipment and air-flow summaries;

- heating, ventilation and air-conditioning equipment efficiencies and control equipment;

- tables showing lighting equipment schedules;

- lighting power density calculations in the design documents;

- lighting controls;

- motor efficiencies and controls;

- findings of the documents review to match with the energy model inputs for the proposed building by using the simulation tool approved by the Bureau;

- scrutinized energy performance index ratio projected at the design stage;

- verify and certify under its seal and authorized signature the items from sub-clause a) to i) of clause ii and iii of sub rule III of the rule 5;

- fill the check list as specified in the Appendix D of the Code and issue correction list in case the design documents of the proposed design of building provide inadequate information or do not meet the requirements of these rules and shall –

- communicate his findings in Form V to the owner of the building under intimation to the concern authority having jurisdiction;

- give specified time to the owner to implement its findings;

- satisfy himself that the communication received from the owner within the specified time, meet the findings and fulfill the shortcomings;

- record his approval and complete the checklist conforming compliance with the Code and these rules, and issue the certificate of approval in Form VI to the owner under intimation to the concerned authority having jurisdiction and State Designated Agency.

- The authority having jurisdiction on receipt of application under sub-rule II of rule 5 for issue of permit for construction of proposed building shall-

- Approve the design and sanction building plan only after it has received a certificate in Form IV from the Empanelled Energy Auditors (Buildings), an undertaking from owner in Form II and, an undertaking from owner in Form III;

- grant permit to erect or re-erect the building or add to or make alterations in the building to carry out the construction works subject to the following conditions in its sanction letter, namely:

- the construction work shall be in accordance with the sanctioned plan and requirement under the Code and these rules;

- the compliance with these rules shall be achieved during construction-in-progress;

- the building shall not be occupied before issuance of occupancy certificate to the owner;

- the authority having jurisdiction may, at any stage, revoke the permit on receipt of non-compliance report from the Empanelled Energy Auditors (Building) or on the notice of any misrepresentation of material facts in the application in respect of the provisions of these rules or the Code after giving a reasonable opportunity of being heard to the owner.

- After receiving the permit, the owner shall –

- give notice of his intention to start the construction work of the building in Form VI;

- undertake construction of energy conservation measures incorporated in the construction documents in terms of sub-clause b) of clause ii of sub-rule I of rule 5;

- have flexibility in constructing the building components and systems covered in the construction documents referred to in clause i of sub-rule I of rule 5 to most effective use of energy by deploying best practices in such components and systems to optimize the energy performance index ratio;

- take the approval of the Empanelled Energy Auditors (Building) before undertaking such construction referred to in clause iii of sub-rule I of rule 5 if the components and systems proposed to be constructed are other than those incorporated in the construction and compliance document.

- The Empanelled Energy Auditors (Building), at construction stage, shall review, verify the specifications of the parameters specified in sub-rule III of rule 5 and, -

- fill out the checklist specified in the Appendix D of the Code, provide comments if the proposed design of building does not meet the construction requirements and specify the shortcomings in compliance to the Code, these rules and sanctioned plan, and shall-

- communicate its shortcomings and finding to the owner;

- give specified time to the owner to implement its findings;

- satisfy himself that the communication received thereafter from the owner meets the specified findings and fulfill shortcomings;

- record his approval and complete the checklist indicating compliance with the Code and these rules, and issue a certificate of compliance in Form VII to the owner under intimation to the authority having jurisdiction;

- where it is determined at any stage that construction is not proceeding in accordance with the sanctioned plan or is in violation of any of the provisions of the Code and these rules, Empanelled Energy Auditors (Building) shall notify the owner, and request for additional information with respect to his findings or on the short comings identified by him as per Form VIII;

- in case the Empanelled Energy Auditors (Building) is satisfied with the additional information provided by the owner, he shall record the same in the certificate of compliance in Form VII and communicate the same to the owner under intimation to the authority having jurisdiction;

- in case the Empanelled Energy Auditors (Building) is not satisfied with the additional information submitted by the owner, he shall report the same to the authority having jurisdiction to ensure that all further construction is stayed until correction has been effectuated and a certificate of compliance has been issued by Empanelled Energy Auditors (Building).

- Every owner shall submit a notice of completion of the building in Form IX to the authority having jurisdiction on the completion of work including the works related to energy conservation measures specified in the sanctioned permit along with the certificate in Form X issued by the Empanelled Energy Auditors (Building) certifying the completion of the building accompanied by-

- the duly completed compliance forms together with check list of various components covered under rule 3 at the completion stage which shall include the followings -

- review of heating, ventilation and air-conditioning component tables for air-handling equipment, refrigeration equipment, condensing equipment, air-flow summaries, tables showing lighting equipment specifications, and tables showing motor specifications;

- inspection of lighting equipment like lamps, ballasts, to confirm fixture wattage and inspection shall include at least random check across according to the type of usage in the building to determine lighting power density;

- review the required lighting controls such as manual switching off perimeter, day lighting circuits, automated occupancy-based control, photo sensor controls, and automated timer-based controls;

- review of coefficient of performance values of installed heating, ventilation and air-conditioning equipment and control equipment;

- review of efficiencies of installed motor and controls;

- review of power factor and power distribution losses;

- review the required check metering and monitoring system.

- a list of the energy related building features in the proposed design, if any, which are different from the sanctioned or standard baseline design;

- all documents and invoices in support of the construction undertaken with respect to all energy conservation measures including insulation, fenestration, heating, ventilation and air-conditioning, lighting and electrical systems, water heating systems of the building.

- the authority having jurisdiction may cross check and verify if the construction has been done in accordance with ECBC.

- If the energy performance index ratio at the completion stage is less than or equal to one as compared to the sanctioned plan of the building, it shall be deemed to have complied with the Code and these rules.

- If there is deviation in the energy performance index ratio of the sanctioned plan that if it is more than one as compared to the sanctioned plan of the building, Empanelled Energy Auditors (Building) shall record its findings in Form XI and communicate the same to the owner and seek compliance of the same through incorporation of additional energy conservation measure. The Empanelled Energy Auditors (Building) shall render technical assistance to the owner to ensure that the proposed design of building becomes compliant with these rules.

- The owner shall neither occupy nor allow any other person to occupy the building or part of the building covered under these rules for any purpose until such building or such part thereof has been granted occupancy certificate under the bye-laws of the authority having jurisdiction.

- The owner shall give notice of completion of the building and seek permission for occupancy.

- The authority having jurisdiction on receipt of such notice by the owner accompanied by a certificate by the Empanelled Energy Auditors (Building), issue the occupancy certificate in Form XII incorporating inter alia the following conditions, namely: -

- that the energy performance of the building shall be monitored and verified by the Manipur Energy Conservation Building Code Implementation Committee;

- that the owner through the Empanelled Energy Auditors (Building) shall submit to the State Designated Agency i.e. Manipur State Power Distribution Company Limited (MSPDCL), an energy performance index report as per Form XIII under intimation to Bureau for two consecutive years after the building has been fully operational;

- in case the energy performance index ratio of the building is more than one, the authority having jurisdiction may issue a provisional occupancy certificate subject to the condition that the owner shall undertake energy audit of the building to identify additional energy conservation measures to achieve the energy performance index ratio of the building approved in the sanctioned plan or permit within a period of three years;

- if the owner fails to achieve the energy performance index ratio as specified in clause iii of sub rule XII of rule 5, within a period of three years from the date of occupancy of the building, the authority having jurisdiction shall place the matter before the Manipur Energy Conservation Building Code Technical Grievances Redressal Committee, which shall hear the owner and the Empanelled Energy Auditors (Building) and make recommendations in the matter accordingly and the authority having jurisdiction shall comply with such recommendations.

- The process shall be continued repeatedly till energy performance index ratio of the building comes to less than one or equal to one and Empanelled Energy Auditors (Building) shall fill and submit the compliance documents, as specified in Appendix D of the Code, of various energy conservation measures at each stage namely, design, construction and completion, to achieve conformity with the Code and these rules.

- The simulation tool referred in sub-clause (i) of clause ii of sub-rule III of rule 5 shall be based on the standard method of test for the evaluation of building energy analysis computer program.

- The owner may approach the Manipur Energy Conservation Building Code Compliant Technical Grievances Redressal Committee for redressal of any grievance under the provisions of these rules.

- Committees-

The State Designated Agency, i.e Manipur State Power Distribution Corporation Limited (MSPDCL), shall constitute-

- Manipur Energy Conservation Building Code Implementation Committee headed by Chief Secretary of Manipur or, his/her nominee and comprising of all stakeholders including a nominee from Bureau, to–

- promote energy efficiency standards through optimization of parameters in the various components and systems of the building in line with the provisions of these rules to enhance the building performance and provide every support to it to make it an effective instrument of promoting energy conservation and energy efficiency in the commercial buildings or establishment;

- forward its recommendations to the Bureau to assist the National Energy Conservation Building Code Implementation Committee to develop and revise energy consumption standards for buildings, in terms of energy performance index, zone-wise – Cold, warm & humid climate zone, classification-wise;

- create awareness about Manipur Energy Conservation Building Code and procedure for erection of Code compliant building;

- promote construction of energy efficient buildings ensuring quality and consistency in their constructions having regard to the climatic conditions and needs of the building projects;

- promote capacity building of building professionals, developers and contractors to promote energy efficient designs of buildings in close co-ordination with authorities having jurisdiction;

- undertake performance review of annual work of all Empanelled Energy Auditors (Building) to check their credentials;

- prepare a summary of violations which shall be provided by the State Designated Agency (SDA) to the Bureau and review such violations for the purpose of evaluating his/her professional skills;

- prepare a yearly report and furnish the same to the Bureau indicating inter alia the progress made in compliance of these rules in the Manipur and the steps taken by the State Designated Agency (SDA) to improve the rate of compliance of Code in the Manipur;

- create data base through compilation of data of energy performance index and its ratio achieved by each building constructed after coming into force of these rules;

- Manipur Energy Conservation Building Code Compliant Technical Grievances Redressal Committee headed by an officer of the Department of Urban Local Bodies/ Department of Town Planning of Manipur, with other members, not exceeding four, nominated by the Manipur Energy Conservation Building Code Implementation Committee who are qualified by experience and training to pass judgment upon matters pertaining to construction of Code compliant building in the state of Manipur, to–

- hear grievance filed by the owner of a Code complaint building within the specified time period given by the authority having jurisdiction relating to the building permit, completion certificate, occupancy certificate of building including determination of the energy performance index ratio at the completion stage and interpretation of these rules or any other grievance arising out of the implementation of the Code and these rules;

- make recommendations to the authority having jurisdiction to reconsider such issue, or for implementation by the authority having jurisdiction, as the case may be.

- Penalty –

The penalty shall be imposed as per Section 26 & 27 of the Energy Conservation Act 2001.

- Responsibilities And Duties of the Owner –

- The owner of the Code compliant building shall carry out the work of the said building in accordance with the requirements of the Code and these rules.

- Every owner shall-

- engage Empanelled Energy Auditors (Building) in development of building design, installation of energy conservation measures and equipment to meet with the requirements of these rules and ensure following, namely: –

- finalize the compliance approach relevant for his building project based on the complexity of the building, budget and time constraints;

- finalize the energy conservation measures as per the Code as amended from time to time having regard to the location of the proposed building;

- to integrate the energy conservation measures in the building design in accordance with the provisions of these rules;

- that drawings, specifications and compliance forms are prepared and energy conservation measures are reflected in the building design documents and submitted to the authority having jurisdiction in compliance with the requirements of the rules accompanied by a certificate specifying the energy performance index ratio of the building by the Empanelled Energy Auditors (Building) that the documents are as per the requirement of these rules;

- notice is given within the validity of sanction to the authority having jurisdiction of his intention to start the construction work at the building site;

- commence the work within the period specified by the authority having jurisdiction from the date of such notice or seek extension of time for starting the construction work, wherever necessary;

- ensure that the designed energy conservation measures are deployed in the construction of the building and installation of its components and systems.

- permit the Empanelled Energy Auditors (Building) and/or authority having jurisdiction to enter the building or premises at any reasonable time for the purpose of inspection to ensure compliance of building works with rules and regulations under the Act;

- give written notice to the authority having jurisdiction intimating the completion of the construction work along with a certificate from the Empanelled Energy Auditors (Building) to the effect that-

- the construction of the building has been done in accordance with the sanction of the building permit;

- all the energy conservation measures have been installed and inspected, and the meet the requirements of the Code and these rules;

- the building design meet with the provisions of the Code and these rules;

- give written notice to the authority having jurisdiction as well as to the State Designated Agency (SDA) in case of termination of the services of Empanelled Energy Auditors (Building) and appointment of other Empanelled Energy Auditors (Building) in its place;

- obtain an occupancy permit from the authority having jurisdiction prior to any occupancy of the building or part thereof after completion of the building;

- report the practical difficulties to the Empanelled Energy Auditors (Building), if any, in carrying out the provisions of these rules, who shall take necessary action in consultation with State Designated Agency (SDA) and Manipur Energy Conservation Building Code Implementation Committee;

- on the receipt of the notice, if any, from the authority having jurisdiction, he shall discontinue such usage within reasonable time as specified in such notice and in no case, he shall disregard the provisions of these rules;

- where he proposes to alter the installation of any system or material or equipment on account of improving the energy efficiency of the building contrary to the system, material or equipment as indicated in the sanction plan, he shall use or install such system or material or equipment after obtaining the necessary approval of the Empanelled Energy Auditors (Building):

Provided that it does not violate the spirit and intent of the provisions of these rules:

Provided further that such change shall not compromise with the building requirements namely, structural stability, safety, health or environmental provisions of Central laws and Manipur laws applicable to the buildings covered under these rules.

- The owner may approach the Manipur Energy Conservation Building Code Compliant Technical Grievances Redressal Committee for redressal of any grievance under the provisions of these rules.

- Role, responsibilities and duties of the Empanelled Energy Auditors (Building) –

The Empanelled Energy Auditors (Building), whose services are engaged by the owner, shall–

- verify and certify –

- the design of the building keeping in view the design criteria, energy goals of the project, energy systems performance verification plan, and the modeling approach;

- the energy conservation measures based on the design approach for the project under consideration;

- construction documents and compliance documents, compliance forms and checklists specified to ensure that the building complies with the Code and these rules;

- energy performance index ratio of the proposed building;

- furnish a certificate under its seal and authorized signature to the effect that drawings, specifications, construction documents, compliance documents and forms prepared covering building envelope, comfort system and controls, lighting and electrical power systems, wherever applicable, and all other Code related documentation prepared for submission to the authority having jurisdiction ensuring compliance with these rules;

- inspect the building works from the design stage to its commissioning stage of buildings including their uses under these rules and based on his certification, the authority having jurisdiction shall issue building permit, approve construction of building, issue completion and occupancy certificates;

- the Empanelled Energy Auditors (Building) shall ensure that none of the professionals or employees working under him/her is engaged in any work in connection with the construction or alteration of the concerned building covered under these rules to ensure that there is no conflict of interest with his/her official duties with the interests of the authority having jurisdiction;

- report to authority having jurisdiction and the State Designated Agency (SDA) on such unusual technical issues that may arise due to issue of building permit or construction of building or during occupancy stage;

- provide inputs to the National and Manipur Energy Conservation Building Code Implementation Committees to facilitate for better implementation of the Code and these rules;

- promote norms and standards specified in the Code.

- Responsibilities and duties of Manipur State Designated Agency (SDA)–

The Manipur State Designated Agency (SDA) established by the Manipur Government under clause (d) of section 15 of the Act, in consultation with Bureau, shall–

- coordinate, regulate and enforce provisions of the Code and these rules for efficient use of energy and its conservation under the Act in the State of Manipur;

- ensure every commercial building or establishment having a connected load of 50 kilowatt (kW) and greater or have a contract demand of 60 kilo-Volt Ampere (kVA) and greater or, be constructed in compliance with these rules;

- monitor the performance of the Empanelled Energy Auditors (Building) to improve the quality, consistency and rate of compliance of these rules with a view to make the cadre of Empanelled Energy Auditors (Building) as effective instruments for promotion of energy efficiency in the building sector in the State of Manipur;

- create a data bank in the State to measure the compliance rates of the Code compliant buildings and accurately account for the energy savings resulting from the compliance of these rules;

- also create a data bank on energy use per square meter of area of the building in Manipur under cold, warm and humid climate;

- take necessary steps to make energy performance index as a measure to comply with these rules in the various categories of buildings and send its recommendations to the Bureau for the formulation of energy consumption norms and standards in respect of various categories of buildings constructed zone-wise in the State;

- arrange conduct site visits, if considered necessary, to determine the accuracy of reporting by Empanelled Energy Auditors (Building) in the State;

- prepare a report on performance of Empanelled Energy Auditors (Building) listing out the projects complying with these rules, projects in violation of compliance with these rules and the level of violation, and provide summary of such violations for each year to the Bureau of Energy Efficiency;

- coordinate with the authority having jurisdiction to amend their building bye-laws incorporating the provisions of these rules for the purpose of construction of buildings in compliance with the Code and these rules;

- provide necessary support to the authority having jurisdiction to conform to the provisions of these rules with regard to matters concerning design construction including energy conservation measures and occupancy for improving the energy performance of Code compliant buildings and effectiveness in compliance of these rules.

- Responsibilities and duties of the authority having jurisdiction and stakeholder departments

- In the state of Manipur, following are the authority having jurisdiction and stakeholder departments:

- Forest Department

- Manipur State Power Distribution Company Ltd.

- Public Works Department (PWD)

- Municipal Administration Housing and Urban Development (MAHUD)

- Manipur Renewable Energy Development Agency (MANIREDA)

- Town Planning Department

- National Institute of Technology Manipur

- The roles and responsibilities of concerned stakeholders for smooth implementation of the Manipur Energy Conservation Building Code (MECBC) in the State are mentioned as follows -

| Sl.no. | Department | Stakeholder involvement | Roles and responsibilities |
| --- | --- | --- | --- |
|  | Forest Department Govt. of Manipur | Climate change component | to synchronize with the State Climate Change Action Plan |
|  | Manipur State Power Distribution Company Ltd. | State Designated Agency (SDA), Manipur | - to amend the Manipur Energy Conservation Building Code (MECBC) as per State’s requirements; - to identify the various stakeholders for the development MECBC; - to form an implementation Committee - to amend the MECBC Rules as per the State’s requirements; - to conduct Capacity building programmes; - to take feedbacks from the stakeholders for MECBC Rules; - to keep in check the energy consumption of the building provided at the design stage through the electricity bills; - record keeping for three years from the completion/ occupancy of the building; - may apply for penalization/ incentive at the later stage, as per the directives received from BEE. |
|  | Public Works Department (PWD) Govt. of Manipur | Implementation agency for Government buildings and other commercial buildings. | to make sure the upcoming Government buildings are MECBC Compliant for the inclusion of Energy-efficient materials in the Schedule of Rates (SoR) |
|  | Department of Municipal Administration Housing and Urban Development (MAHUD) | For Bye-laws and administration. | to make necessary preparations for updating the Bye-laws with MECBC norms/directives. |
|  | Manipur Renewable Energy Development Agency (MANIREDA) | Renewable Energy partner | - Provide and check if the building meets the mandatory renewable energy requirement for MECBC compliance |
|  | Manipur Town Planning Department | Verification of the building profiles. | - Verification and approval agency for buildings above 1000 sqm. Integrate MECBC in building approval process (OBPS) |
|  | National Institute of Technology Manipur | Knowledge partner (technical institute of the State) | - Responsible for R&D in NECBC at State level - Responsible for training the students in grassroot level - May be developed as a Centre of Excellence. |

- Authorities having jurisdiction shall ensure the implementation of Energy Conservation Building Code in the commercial buildings, having connected load of 50 kW or above or, a contract demand of 60KVA or above in buildings listed in sub rule I&II of rule 3.

- The concerned stakeholder departments shall ensure undertaking from owner, Architect/Firm responsible for the construction of the building, at the change of land use stage.

- The concerned stakeholder department shall ensure undertaking from owner and certificate from Empanelled Energy Auditors (Building) before awarding building permit.

- At construction stage, the works / certificate of the Empanelled Energy Auditors (Building) shall be verified randomly at site by the officials of the concerned stakeholder departments. If any discrepancy noticed regarding certification of the works by the Empanelled Energy Auditors (Building), the concerned stakeholder departments shall intimate to owner/ architect as per the process of implementation of ECBC Compliance

- Miscellaneous. –

- The use of any energy conservation measures or method or design or construction not specifically specified under these rules shall not be prevented by the authority having jurisdiction if such energy conservation measures or method or design or construction is found to be satisfactory by the Manipur Energy Conservation Building Code Compliant Technical Grievance Redressal Committee and such energy conservation measures or method or design or construction assist the owner in optimizing the energy performance index ratio in the use of energy on its occupancy.

- The Code shall be reviewed periodically, at least once in five years, to determine the need for revision or withdrawal of standards specified in the Code, and such standards which in the opinion of the Bureau need no revision or amendment shall be reaffirmed.

- In case of the owners or their authorized agents violate the provisions of the Manipur Energy Conservation Building Code (MECBC), or any provisions of these directions, action shall be taken against such owner or his authorized agent by the authority under jurisdiction which may include cancellation or denial of the completion or occupancy certificate or any other penalty.

- The concerned stakeholder departments shall maintain a register of the buildings to which these directions are applicable and send a quarterly progress report of MECBC Compliant buildings to the State Designated Agency (SDA) containing therein a list of the buildings sanctioned or completion certificates awarded after coming into force of these directions.

- All the concerned stakeholder departments shall amend its building Bye-Laws within a period of six months from the publication of these directions in the State Gazette for strict enforcement of the Manipur Energy Conservation Building Code (MECBC) and for prescribing penalties in case of its violation.

| LIST OF FORMS |  |  |  |
| --- | --- | --- | --- |
| Sr.No. | Form No. | Reference Clause | Description |
| 1 | Form I | Sub-rule I of Rule 5 | Application for seeking building permit in respect of erection/ re-erection/making alteration in the Manipur Energy Conservation Building Code Compliant Building. |
| 2 | Form II | Clause i of Sub-rule I of Rule 5; Clause i of Sub-rule IV of Rule 5 | Undertaking by owner for construction of the Manipur Energy Conservation Building Code compliant building. |
| 3 | Form III | Clause ii of Sub-rule I of Rule 5; Clause i of Sub-rule IV of Rule 5; | Undertaking by registered Architect/ BEE Empanelled ECBC expert professional or firm/ MSPDCL Empanelled ECBC Design professional or firm for designing and construction of the Manipur Energy Conservation Building Code compliant building. |
| 4 | Form IV | Clause iv of Sub-rule I of Rule 5; Clause i of Sub-rule IV of Rule 5; | Certificate from Empanelled Energy Auditors (Buildings) to be enclosed with the application for Building Permit for Energy Conservation Building Code compliant building |
| 5 | Form V | Sub-clause a) of Clause v of Sub-rule III of Rule 5 | Certificate of Inspection by Empanelled Energy Auditors (Buildings) on review of Building Permit Application in respect of the proposed building - Communication of omissions and non-compliance to owner |
| 6 | Form VI | Clause vi of Sub-rule III of Rule 5 | Certificate of Inspection by Empanelled Energy Auditors (Buildings) on review of building permit application enclosing construction documents and compliance forms in respect of Energy Conservation Building Code compliant building- no objection certificate |
| 7 | Form VII | Clause ii of Sub-rule VI of Rule 5 | Certificate of Inspection by Empanelled Energy Auditors (Buildings) on review of construction works enclosing construction documents and compliance forms in respect of Energy Conservation Building Code compliant building- Issue of certificate of compliance |
| 8 | Form VIII | Clause iii of Sub-rule VI of Rule 5 | Certificate of Inspection by Empanelled Energy Auditors (Buildings) on review of construction works enclosing construction documents and compliance forms in respect of Energy Conservation Building Code compliant building- Issue of certificate of non-compliance |
| 9 | Form IX | Sub-rule VII of Rule 5 | Notice of completion |
| 10 | Form X | Sub-rule VII of Rule 5 | Certificate of Inspection by Empanelled Energy Auditors (Buildings) on review of completion of construction works enclosing construction documents and compliance forms in respect of Manipur Energy Conservation Building Code compliant building- Issue of certificate of compliance |
| 11 | Form XI | Sub-rule IX of Rule 5 | Certificate of Inspection by Empanelled Energy Auditors (Building) on review of completion of construction works in respect of Energy Conservation Building Code compliant building-Communication of omissions and non-compliance to owner |
| 12 | Form XII | Sub-rule XII of Rule 5 | Occupancy Certificate, to be issued by Authority having Jurisdiction in their occupancy certificate |
| 13 | Form XIII | Clause ii of Sub-rule XII of Rule 5 | Energy performance index Report Submission by Empanelled Energy Auditors (Buildings) to the State Designated Agency (SDA) after the building has become fully operational |

Form I

[See Sub-rule I of Rule 5]

[Application for seeking building permit in respect of erection/ re-erection/ making alteration in the Manipur Energy Conservation Building Code Compliant Building]

To,

Authority having jurisdiction,

Name of the city,

Name of the State         				                     		 Date: __/__/____

Subject: Application for erection of Manipur Energy Conservation Building Code compliant building in premises of Plot no.____ Block No. ______Scheme________ Street________ Name of the town/city

Sir,

I/We the undersigned hereby give you notice of my intention to erect/ re-erect/ alter Manipur Energy Conservation Building Code Compliant Building under the Manipur Energy Conservation Building Code Rules, 2025 in the premises of plot No.______________ Lane No.___________ Scheme________ and request for issue of building permit for the construction of the Manipur Energy Conservation Building Code compliant building. The following documents are enclosed.

- Construction Documents and Compliance Forms together with check-lists incorporating the installation of Energy Conservations Measures specified in the aforesaid rules.

- The Construction Documents and Compliance Forms together with check-lists have been verified by Shri----------------------- Regd. number__________ Empanelled Energy Auditor (Building). An undertaking certificate in Form II duly signed and sealed in this regard is enclosed.

Yours faithfully,

(Name of the owner)

Address:

Tel. No./Mobile No.:

Form II

[See Clause i of Sub-rule I of Rule 5; Clause i of Sub-rule IV of Rule 5]

[Undertaking by owner for construction of the Manipur Energy Conservation Building Code compliant building]

To

Authority having jurisdiction,

Name of the city,

Name of the State         							Date: __/__/____

Sir/Madam,

I/We am/are the owner(s) of the proposed Manipur Energy Conservation Building Code Compliant Building in the premises of Plot No. ________________________Lane no.___________ Scheme___________ in the town/city of ___________, Manipur wherein I/We intend to construct the building. The proposed building on completion of construction shall have a connected load/contract demand of 50 kW/60kVA or above and is proposed to be constructed to use or intended to be used for commercial purposes.

I/We undertake that the aforesaid building shall be constructed in accordance with the provisions of the Manipur Energy Conservation Building Code Rules 2025. In case any deviation is noticed during the construction of the Building, I/we shall indemnify the loss to the authority having jurisdiction. In case of any deviation in the proposed design, the same will be conveyed and approved by the Empanelled Energy Auditors and submitted to Authority having Jurisdiction.

I/ we further undertake that the information supplied in the enclosed drawings, construction and compliance documents and the application is accurate to the best of my/our knowledge and if any of the information supplied is found to be incorrect and such information result in loss to the Central or the State Government or any other authority under them. I/ we undertake to indemnify such loss.

Enclosed: Form IV

Yours Faithfully

(Signature)

(Name of the owner(s)):

Address:

Mobile No/ Tel No:

Form III

[See Clause ii of Sub-rule I of Rule 5; Clause i of Sub-rule IV of Rule 5]

[Undertaking by registered Architect/ BEE Empanelled ECBC expert professional or firm/ MSPDCL Empanelled ECBC Design professional or firm for designing and construction of the Manipur Energy Conservation Building Code compliant building]

I/We am/are the registered Architect/ BEE Empanelled ECBC expert professional or firm/ MSPDCL Empanelled ECBC Design professional or firm having License No/ Registration No. ________________________ being assigned the contract for designing and construction of the proposed building of the owner Ms/Shri ________________________in the premises of Plot No. ________________________Lane no.___________ Scheme___________ in the town/city of ___________, Manipur.

I/We undertake the significance of Manipur Energy Conservation Building Code. I/We hereby undertake that the Manipur Energy Conservation Building Code shall be considered in the aspect of designing and construction works of the proposed building in the aforementioned plot under the guidance of a certified Energy Auditor (Building) appointed by the owner.

I/ we further undertake that the information supplied in the enclosed drawings, construction and compliance documents and the application is accurate to the best of my/our knowledge in accordance with the bye-laws of the authority and the provisions of the Manipur ECBC 2025. In case any deviations are noticed during the construction of the Building, I/We shall rectify the deviations with the permission of authority having jurisdiction.

(Signature)

Name of the Architect/ Authorized signatory

License no/ registration no.

Office Address:

Seal

Date_/_/_

Form IV

[Clause iv of Sub-rule I of Rule 5; Clause i of Sub-rule IV of Rule 5]

[Certificate from Empanelled Energy Auditor (Building) to be enclosed with the building permit for the Manipur Energy Conservation Building Code compliant building]

Certificate

Compliance Certificate No __________

I/We am/are Empanelled Energy Auditor (Building) having registration No _______under the Energy Conservation Act 2001 (52 of 2001) and am authorized to scrutinize and verify the design of Manipur Energy Conservation Building Code (MECBC) 2025 compliant building. I/We certify that –

- I/We have scrutinized the construction documents, undertaking given by the owner duly signed by the owner/Architect showing all the pertinent data and features of the building, equipment and systems in sufficient details covering building envelop, heating, ventilation and air-conditioning, service hot water, lighting and electrical power in accordance with Municipal Bye-laws and with the Manipur Energy Conservation Building Code (MECBC) Rules 2025 in respect of building proposed to be constructed on plot on _________lane no_________  scheme_________ in the city of _________ in the State of Manipur;

- I/We have scrutinized the compliance forms with the check- lists to ensure compliance with the Bye-laws and the MECBC Rules 2025.

- The compliance documents have been duly inspected by the undersigned.

- The energy performance index ratio of the building design as per compliance documents, at the design stage is equal to or less than one and is therefore in compliance with the MECBC Rules 2025.

- It is certified that all required scrutiny and verification of the documents submitted have been carried out diligently, truthfully and all reasonable professional skill, care and diligence have been taken in scrutinizing and verifying the drawings of the buildings and compliance forms together with check–lists covering the various components of the MECBC Rules 2025.

- The contents of all the documents submitted along with the application are a true representation of the facts and nothing has been concealed.

There is no objection for issue of building permit in respect of the aforesaid proposed building in so far as requirements of MECBC Rules 2025 are concerned.

Authorized Signature:

Name of the Empanelled Energy Auditor (Building):

Registration No /Seal:

Form V

[See Sub-clause a) of Clause v of Sub-rule III of Rule 5]

[Certificate of Inspection by Empanelled Energy Auditor (Building) on review of Building Permit Application in respect of the proposed building - Communication of omissions and non-compliance to owner]

To,

Shri _____________

Address________							Date: __/__/____

Subject: Application for erection of proposed Building in premises of Plot no. _____Lane No._____ Scheme________ Street________ Name of the town/city- Details of omission/ non-compliance with the Manipur Energy Conservation Building Code Rules 2025 on design stage inspection.

Sir,

I/We,…………………….(Name), being an authorized Empanelled Energy Auditors (Building) vide order No. ___________________  hereby  state  I/we have reviewed and verified the  undertaking given by you and have inspected the construction documents, compliance forms, check-lists, submitted along with building permit application in respect of the various elements specified in sub-rule III of rule 5 of the various components of the proposed building in respect of the subject building and inform that the following omission/ non-compliance have been discovered on inspection–

It is requested that the necessary energy conservation measure in consultation with your design team be carried out in order to bring them in compliance with the Manipur Energy Conservation Building Code Rules 2025. You are accordingly requested to take corrective action within a period of one month from the date of issue of this letter. Further action on your application for issue of building permit shall be taken after satisfactory compliance of the aforesaid omission/non-compliance.

Signature:

Name of Empanelled Energy Auditors (Building):

Registration no./Mobile no.

Seal:

Form VI

[See Clause vi of Sub-rule III of Rule 5]

[Certificate of Inspection by Empanelled Energy Auditors (Building) on review of building permit application enclosing construction documents and compliance forms in respect of Manipur Energy Conservation Building Code compliant building—no objection certificate]

Certificate No

Date: __/__/________

I/We,…………………….(Name),  being an authorized/ Empanelled Energy Auditors (Building) vide order No.___________________ hereby state I/we have reviewed and verified the undertaking given by the owner and have inspected the construction documents, compliance forms, check-lists, submitted along with building permit application in respect of the various  elements of the proposed Manipur Energy Conservation Building Code compliant building in the premises of Plot No.______________ Lane No.___________ Leikai________ Town/City ________ State of__________ and certify that the

- The omission/non-compliance pointed out by the undersigned in the certificate of Inspection dated have been complied satisfactorily;

- The energy performance index ratio calculation match with the data given in the aforesaid documents and is in compliance with the Manipur Energy Conservation Building Code Rules 2025.

I/We further certify that –

- all reasonable professional skill, care, and diligence have been taken in verifying the construction document and compliance forms in respect of the various elements of the components covered in Manipur Energy Conservation Building Code Rules 2025 and contents thereof are a true representation of the facts and meet the requirements of MECBC Rules 2017.

- There is no objection for the issue of building permit in respect of the aforesaid proposed building in so far as requirements of Manipur Energy Conservation Building Code Rules 2025 are concerned.

The check-list duly completed and signed by the undersigned is enclosed.

Signature:

Name of Empanelled Energy Auditors (Building):

Registration No/Mobile no.

Seal:

Form VII

[See Clause ii of Sub-rule VI of Rule 5]

[Certificate of Inspection by Empanelled Energy Auditors (Building) on review of construction works enclosing construction documents and compliance forms in respect of Manipur Energy Conservation Building Code compliant building-- Issue of certificate of compliance]

To,

The Owner, 							Compliance Certificate No

Address 							Date: __/__/__________

I/We,…………………….(Name),  being an authorized/ Empanelled Energy Auditors (Building) vide  order No.___________________ hereby state I/we have reviewed the undertaking given by the owner, energy conservation measures installed during the construction works and have reviewed the construction documents, compliance forms, check-lists, submitted along with progress in construction works in respect of the various elements of the components referred to in sub-rule VI of Rule 5 of the proposed Manipur Energy Conservation Building Code compliant building in the premises of Plot No.______________ Lane No.___________ Leikai________ Town/City ________ State of__________ and certify that the energy performance index ratio calculation match with the data given in the aforesaid documents;

I/We further certify that all reasonable professional skill, care, and diligence have been taken in verifying the construction document and compliance forms in respect of the various elements of the components covered in Manipur Energy Conservation Building Code Rules 2025 and contents thereof are a true representation of the facts and meet the requirements of MECBC Rules 2017.

The check-list duly completed and signed by the undersigned is enclosed.

Signature:

Name of Empanelled Energy Auditors (Building):

Registration No/Mobile no.

Seal:

Copy to:

- Commissioner, Authority having Jurisdiction

- Managing Director, State Designated Agency

Form VIII

[See Clause iii of Sub-rule VI of Rule 5]

[Certificate of Inspection by Empanelled Energy Auditors (Building) on review of construction works enclosing construction documents and compliance forms in respect of Manipur Energy Conservation Building Code compliant building-- Issue of certificate of compliance]

To,

The Owner, 							Compliance Certificate No

Address 							Date: __/__/__________

Subject: Erection of Manipur Energy Conservation Building Code compliant building on premises of Plot No.______________ Lane No.___________ Leikai________ Town/City ________ - Communications of findings by the Empanelled Energy Auditors (Building)

Sir

I/We,…………………….(Name),  being an authorized/ Empanelled Energy Auditors (Building) vide  order No.___________________ hereby state I/we have reviewed the undertaking given by the owner, energy conservation measures under construction and have reviewed the construction documents, compliance forms, check-lists, submitted along with progress in construction works in respect of the various elements of the components of the proposed Manipur Energy Conservation Building Code compliant building in the premises of Plot No.______________ Lane No.___________ Leikai________ Town/City ________ State of__________ and have to state that the construction has not proceeded in accordance with the sanctioned plan and has deviated/is deviating from the following provisions of Manipur Energy Conservation Building Code Rules 2025, namely: -

- None of the above deviations are covered in the best practices approved by the Manipur Energy Conservation Building Code Implementation Committee.

- The building owner is requested to rectify the above deviations or take the approval of the Manipur Energy Conservation Building Code Technical Grievance Redressal Committee.

- The building owner after obtaining the approval provide in para 3 above or rectifying the deviations notified in para 1 above may inform the undersigned of the action taken in the matter within one month from the date of approval obtained or rectification completed along with the updated check-list to enable me to inspect the works in connection with the issue of certificate of approval provided in clause iv of sub-rule VI of rule 5 of the Manipur Energy Conservation Building Code Rules 2025.

Signature:

Name of Empanelled Energy Auditors (Building):

Registration No/Mobile no.

Seal:

Copy to:

- Commissioner, Authority having Jurisdiction

- Managing Director, State Designated Agency

Form IX

[See Sub-rule VII of Rule 5]

Notice of completion

To,

Authority having jurisdiction,

Address    							Date: __/__/_________

Subject: Construction of Manipur Energy Conservation Building Code compliant building on plot No. __________ in Lane No.__________ Leikai___________ Notice of completion of construction of Manipur Energy Conservation Building Code compliant works.

Sir,

I/We hereby give notice that the erection of the building on plot No…………Lane No. ……………. including execution and implementation of the energy conservation measures have been completed in accordance with the plans sanctioned vide your office communication No...................... dated………........... The following documents are enclosed: -

- A certificate of inspection on completion of the aforesaid building from (Name) _____________ Empanelled Energy Auditors (Building) vide Municipal Authority Order No. ____________ dated _________

The building is fit for use for which it has been erected/re-erected/constructed. It is requested that permission to occupy or use the aforesaid building may be granted.

Yours faithfully,

(Signature)

Name of the owner:

Plot No.:

Lane No.:

Address:

Form X

[See Sub-rule VII of Rule 5]

[Certificate of Inspection by Empanelled Energy Auditors (Building) on review of completion of construction works enclosing construction documents and compliance forms in respect of Manipur Energy Conservation Building Code compliant building- Issue of certificate of compliance]

To,

Name…………

Owner of the Building, 						Compliance Certificate No

Address							Date: __/__/__________

Subject: Completion of Construction Works in respect of Manipur Energy Conservation Building Code compliant building-

Certificate

I/We,…………………….(Name(s)), being the authorized/ Empanelled Energy Auditors (Building) vide order No. ___________________ hereby state that I have verified the undertaking given by the owner, and have inspected the construction documents, compliance forms, check-lists, submitted on completion of building of the proposed Manipur Energy Conservation Building Code compliant building in the premises of plot No.______________ Lane No.___________ Leikai________ town/City________ state of__________ and certify that:

- The works covered under the Manipur Energy Conservation Building Code Rules 2025 have been completed to the best of my satisfaction. The details of the various components/system completed as per MECBC Rules 2025 are given below:

| Name of the component | Dated |
| --- | --- |

- The energy performance index ratio of the said building match/with the data given in the aforesaid compliance documents specified in para 1 above.

- A list of the energy conservation measures deployed in the construction of aforesaid building enclosed. Necessary approvals required have been taken by the owner.

- The building in my/our view meets the requirements of Manipur Energy Conservation Building Code Rules compliant building and is fit for occupancy for which it has been erected.

- I further certify that all reasonable professional skill, care, and diligence have been taken in verifying the construction document and compliance forms in respect of the various elements of the components covered in the MECBC Rules 2025 and contents thereof are a true representation of the facts and meet the requirements of the MECBC Rules 2025.

- The check-list duly completed, signed sealed by the undersigned is enclosed.

Signature:

Name of Empanelled Energy Auditors (Building):

Registration No/ Seal:

Copy to:

- Competent Authority/ Authority having Jurisdiction

- Managing Director, State Designated Agency, MSPDCL

Form XI

[See Sub-rule IX of Rule 5]

[Certificate of Inspection by Empanelled Energy Auditors (Building) on review of completion of construction works in respect of Manipur Energy Conservation Building Code compliant building- Communication of omissions and non-compliance to owner]

To,

Shri ___________,					Non-Compliance Certificate No

Address							Date: __/__/_________

Subject: Application for erection of Manipur Energy Conservation Building Code Compliant Building in premises of plot no. _____Lane No. _____ Leikai________ street________ name of the town/city- details of omission /non-compliance with the Manipur Energy Conservation Building Code Rules 2025 on design/completion stage inspection.

Sir,

I/We …………………. (Name), being an authorized/ Empanelled Energy Auditors (Building) vide order No.___________________ hereby state I/we have reviewed and verified the undertaking given by you and have inspected the construction documents, compliance forms, check-lists, submitted on completion of the proposed Manipur Energy Conservation Building Code compliant building in respect of the subject building and inform that the following omission/non-compliance have been found on inspection –

You are accordingly requested to take corrective action within a period of three months from the date of issue of this letter. Further action on your application for issue of Completion Certificate shall be taken after satisfactory compliance of the aforesaid omission/non-compliance.

Signature:

Name of Empanelled Energy Auditors (Building):

Registration No/ Seal:

Copy to:

- Competent Authority/ Authority having Jurisdiction

- Managing Director, State Designated Agency, MSPDCL

Form XII

[See Sub-rule XII of Rule 5]

Occupancy Certificate

(To be issued by Authority having jurisdiction in their occupancy certificate)

To,

Name of the owner….

Address……

Subject: Issue of Occupancy Certificate

Sir,

With reference to your notice of completion of a Manipur Energy Conservation Building Code compliant building construction dated………. on plot no…………… lane no…………… situated at …………. I/we hereby certify that the said building as per description annexed on plot No. _______ lane No.____ leikai_____ whose plans were sanctioned vide No …………dated………has been inspected with reference to requirements of Manipur Energy Conservation Building Code Rules 2025.

- The building is declared fit for occupancy as follows:

Climate Zone	         	24-Hour use Building / Regular Building

Hospitals/Hotels/Call Centre/Other Building Types

| Annexure |  |  |
| --- | --- | --- |
|  | Description of the building |  |
| Ground Floor | Usage | connected load |
| 1st floor | ……. | ……. |
| 2nd floor | ……. | ……. |

- The energy performance index ratio of the building on the completion stage is as per the sanction plan. It has been decided by the authority having jurisdiction in consultation with the State Designated Agency (SDA) that the building “fit for occupancy” as specified above, subject to the condition that the owner shall undertake energy audit of the building and identify additional energy conservation measures to achieve the compliance with the energy performance index of the building approved in the sanctioned plan.

- The energy performance of the building shall be monitored and verified by the Manipur Energy Conservation Building Code committee for the next two years.

Enclosed: copy of certified completion plan.

Yours faithfully,

(Signature of building official)

Authority having jurisdiction

Copy to: Managing Director, State Designated Agency, MSPDCL

Form XIII

[See Clause ii of Sub-rule XII of Rule 5]

[Energy performance index Report Submission by Empanelled Energy Auditors (Building) to State Designated Agency/Authority having Jurisdiction after the building has become fully operational]

To,

The Managing Director,

MSPDCL/ Authority having Jurisdiction

State Designated Agency

Address

Subject: Energy performance index Report for Manipur Energy Conservation Building Code compliant building constructed on Plot no. _____ Lane No. ______Leikai ________ Street________ Name of the town/city-Communication by the Empanelled Energy Auditors (Building).

Sir,

I/We, ……………………. (Name), being the authorized/ Empanelled Energy Auditors (Building) vide order No.___________________ hereby state that I/we have	reviewed the undertaking given by the owner, energy consumption for year………. of the proposed building of type ………………….in the premises of plot No.______________ Lane No.__________ Leikai________ Town/City________ State of__________ and certify that the energy performance index ratio is………. which is less than or equal to one. The EPI report is enclosed.

I further certify that all reasonable professional skill, care, and diligence have been taken to verify the energy consumption of the aforesaid building.

Copies of the electricity bills have been enclosed for your reference.

Encl: Energy performance index ratio report as specified in the Appendix D of the Code.

Yours faithfully,

Signature:

Name of Empanelled Energy Auditors (Building)

Registration No/ Seal:

CC: 	Director, Buildings Programme, Bureau of Energy Efficiency,

4th Floor, Sewa Bhavan, R K Puram, New Delhi– 110 066